MEMORANDUM

| To: | Charlie Refvem, Lecturer, Department of Mechanical Engineering, Cal Poly SLO |  |
| --- | --- | --- |
|  | crefvem@calpoly.edu |  |
| From: | Quinn Patterson |
|  | qpatters@calpoly.edu |
|  | Jake Rowen |
|  | jhrowen@calpoly.edu |
| Team: Date: | 23 3/05/26 |  |
| RE: | Lab 0x06 Memo |  |

Table of Contents

- Memo Body

- Source Code

Memo Body

The state estimator will be useful tool to control Romi’s speed when not in line following mode. We used the IMU mode to access gyroscope and accelerometer data. These measurements coupled with encoder counts and motor command voltages allow us to make useful state predictions of Romi. To get a strong baseline about where Romi should be in testing, we used our line follower code to have Romi trace a circle. Our predicted location was fairly accurate, only deviating at beginning of the data. This deviation is to be expected since the initial input matrix for the state estimator is all zeros, meaning the estimator needs data accurately estimate the state. Our track path can be viewed in the figure below. The observed state consists of Arc length, heading angle, and rotational velocities for each motor. The first two can be used to estimate the location of the robot, as we did in our graph.

Figure 1. Romi State Estimation of Circular Path

We implemented two new tasks: a read sensor task and state estimator task. Read sensor pulls data from the IMU, Encoders, and Motors and stores them in shares for use in our u* matrix. They are then transferred via shares to the state estimator task which handles our estimation. An updated diagram of our tasks and FSMs can be viewed below. A class was written for the IMU that is references in the read sensor task. Additionally, two new queues are used for storing arc length and heading angle data from the state estimation task, which we used to create the above plot. The parameters from most recent estimated state are also stored in shares however are currently unused in our code.

Figure 2. Task Diagram

Figure 3. FSM for Sensor Task

Figure 4. FSM for Estimator Task

Source Code

IMU_class.py:

import pyb

from pyb import I2C

from micropython import const

from time import sleep_ms

import os

# --- I2C address options ---

ADDR_A = const(0x28)

ADDR_B = const(0x29)

# --- Registers (subset) ---

CHIP_ID     = const(0x00)

PAGE_ID     = const(0x07)

ACC_DATA_X_LSB   = const(0x08)

MAG_DATA_X_LSB   = const(0x0E)

GYR_DATA_X_LSB   = const(0x14)

EULER_H_LSB      = const(0x1A)

QUA_W_LSB        = const(0x20)

LIA_DATA_X_LSB   = const(0x28)

GRV_DATA_X_LSB   = const(0x2E)

ACC_OFF_X_LSB = const(0x55)

MAG_OFF_X_LSB = const(0x5B)

GYR_OFF_X_LSB = const(0x61)

ACC_RAD_LSB = const(0x67)

CALIB_STAT  = const(0x35)

OPR_MODE    = const(0x3D)

PWR_MODE    = const(0x3E)

SYS_TRIGGER = const(0x3F)

UNIT_SEL    = const(0x3B)

# --- Operation modes ---

MODE_CONFIG = const(0x00)

MODE_NDOF   = const(0x0C)   # fusion: accel+gyro+mag

MODE_IMU    = const(0x08)

# --- Power modes ---

POWER_NORMAL = const(0x00)

class IMU:

def __init__(self, i2c, addr=ADDR_A):

# Using I2C2, device address is 0x28

self._i2c = i2c

self._addr = addr

self._buf = bytearray(8)

def readbyte(self, reg):

return self._i2c.mem_read(1, self._addr, reg)[0]

def readbytes(self, n, reg):

return self._i2c.mem_read(n, self._addr, reg)

def writebytes(self, val, reg):

self._i2c.mem_write(bytes([val & 0xFF]), self._addr, reg)

def write2bytes(self, val, reg):

val &= 0xFFFF

lsb = val&0xFF

msb = (val >> 8) & 0xFF

self.writebytes(lsb,reg)

self.writebytes(msb,reg+1)

@staticmethod

def combine(lsb, msb):

v = (msb<<8) | lsb

return v-65536 if v & 0x8000 else v

def read_vector(self, startreg, unit_scale):

data = self.readbytes(6, startreg)

x = self.combine(data[0], data[1]) / unit_scale

y = self.combine(data[2], data[3]) / unit_scale

z = self.combine(data[4], data[5]) / unit_scale

return x,y,z

def set_mode(self, mode):

self.writebytes(mode, OPR_MODE)

sleep_ms(30)

def initialize(self, mode=MODE_IMU, reset=True, units = 0x06):

# units 0x00 gives m/s^2, degrees/sec, Celcius

self.writebytes(0x00, PAGE_ID)

sleep_ms(5)

self.set_mode(MODE_CONFIG)

# Power Mode: Normal

self.writebytes(POWER_NORMAL, PWR_MODE)

sleep_ms(20)

self.writebytes(units, UNIT_SEL) # select standard units

sleep_ms(10)

# Calibration data

fname = "calibration.txt"

try:

os.stat(fname)

with open(fname, 'r') as file:

constants = file.read().strip().split(',')

vals = [int(c) for c in constants if c!=""]

ACC = vals[0:3]

MAG = vals[3:6]

GYR = vals[6:9]

ACC_RAD = vals[9]

MAG_RAD = vals[10]

self.write_coeff(ACC, MAG, GYR, ACC_RAD, MAG_RAD)

except OSError:

self.set_mode(MODE_NDOF)

while(True):

stat = self.calibration_status()

print(stat)

if all(bit == 3 for bit in stat[1:]):

print("Calibration complete")

break

self.set_mode(MODE_CONFIG)

ACC_OFF, MAG_OFF, GYR_OFF, ACC_RAD, MAG_RAD = self.get_coeff()

lst = list(ACC_OFF) + list(MAG_OFF) + list(GYR_OFF) + [ACC_RAD, MAG_RAD]

with open(fname, 'w') as file:

file.write(','.join(str(c) for c in lst)+ "\n")

self.set_mode(mode) # enter IMU mode

def calibration_status(self, reg=CALIB_STAT):

# data in the order: system, gyro, accelerometer, magnetometer

cbt = self.readbyte(reg)

system = (cbt >> 6) & 0b11

gyro = (cbt >> 4) & 0b11

acc = (cbt >> 2) & 0b11

mag = cbt & 0b11

return [system, gyro, acc, mag]

def get_coeff(self,reg=ACC_OFF_X_LSB):

# format: [x data, y data, z data]

raw = self.readbytes(22, reg)

ACC_OFF = (self.combine(raw[0],raw[1]) & 0xFFFF,self.combine(raw[2],raw[3]) & 0xFFFF,self.combine(raw[4],raw[5]) & 0xFFFF)

MAG_OFF = (self.combine(raw[6],raw[7]) & 0xFFFF,self.combine(raw[8],raw[9]) & 0xFFFF,self.combine(raw[10],raw[11]) & 0xFFFF)

GYR_OFF = (self.combine(raw[12],raw[13]) & 0xFFFF,self.combine(raw[14],raw[15]) & 0xFFFF,self.combine(raw[16],raw[17]) & 0xFFFF)

ACC_RAD = self.combine(raw[18],raw[19]) & 0xFFFF

MAG_RAD = self.combine(raw[20],raw[21]) & 0xFFFF

return ACC_OFF,MAG_OFF,GYR_OFF,ACC_RAD,MAG_RAD

def write_coeff(self, ACC, MAG, GYR, ACC_RAD, MAG_RAD, reg=ACC_OFF_X_LSB):

self.write2bytes(ACC[0], reg)

self.write2bytes(ACC[1], reg+2)

self.write2bytes(ACC[2], reg+4)

self.write2bytes(MAG[0], reg+6)

self.write2bytes(MAG[1], reg+8)

self.write2bytes(MAG[2], reg+10)

self.write2bytes(GYR[0], reg+12)

self.write2bytes(GYR[1], reg+14)

self.write2bytes(GYR[2], reg+16)

self.write2bytes(ACC_RAD, reg+18)

self.write2bytes(MAG_RAD, reg+20)

def euler(self, reg=EULER_H_LSB):

angles = self.read_vector(reg, 900.0)

return angles

def ang_v(self, reg=GYR_DATA_X_LSB):

rates = self.read_vector(reg, 900.0)

return rates

def accel(self, reg=ACC_DATA_X_LSB):

accel = self.read_vector(reg, 100)

return(accel)

task_read_sensor.py:

from IMU_class import IMU

from encoder_class import Encoder

from task_share import Share

from micropython import const

from motor_class import Motor

S1_RUN = const(1)

class read_sensors:

def __init__(self, IMU, EncoderL, EncoderR, MotorL, MotorR, sL, sR, yaw, yaw_rate, voltageL, voltageR):

self.IMU : IMU = IMU

self.EncoderL : Encoder = EncoderL

self.EncoderR : Encoder = EncoderR

self.MotorL : Motor = MotorL

self.MotorR: Motor = MotorR

self.sL : Share = sL

self.sR : Share = sR

self.yaw : Share = yaw

self.yaw_rate : Share = yaw_rate

self.voltageL : Share = voltageL

self.voltageR : Share = voltageR

self._state = S1_RUN

def run(self):

while(True):

if self._state == S1_RUN:

self.sL.put(self.EncoderL.get_position())

self.sR.put(self.EncoderR.get_position())

self.yaw.put(self.IMU.euler()[0])

self.yaw_rate.put(self.IMU.ang_v()[2])

self.voltageL.put(self.MotorL.get_voltage())

self.voltageR.put(self.MotorR.get_voltage())

yield self._state

task_estimate_state.py:

from IMU_class import IMU

from encoder_class import Encoder

from ulab import numpy as np

from micropython import const

from task_share import Share, Queue

S0_WAIT = const(0)

S1_RUN = const(1)

#shareEstimator is a share that sets the state of this task accordingly

class state_est_task:

def __init__(self, sL, sR, voltageL, voltageR, yaw, yaw_rate, shareEstimator, estS, estPsi, estOmegaL, estOmegaR, dataValsArc, dataValsPsi):

self.sL : Share = sL

self.sR : Share = sR

self.voltageL : Share = voltageL

self.voltageR : Share = voltageR

self.yaw : Share = yaw

self.yaw_rate :Share = yaw_rate

self.dataValsArc : Queue = dataValsArc

self.dataValsPsi : Queue = dataValsPsi

self._state = S0_WAIT

self._shareEst : Share = shareEstimator

self._estS : Share = estS

self._estPsi : Share = estPsi

self._estOmegaL : Share = estOmegaL

self._estOmegaR : Share = estOmegaR

self._encLInitial = 0

self._encRInitial = 0

self._headingInitial = 0

self._xhat = np.zeros((4,1), dtype = np.float) # initialize state vector

self._AD = np.array([[0.7427, 0, 0.2494, 0.2494],[0, 0.0061, 0, 0],[-0.1212, 0, 0.3192, 0.3095],[-0.1212, 0, 0.3095, 0.3192]],dtype = np.float)

self._BD = np.array([[0.1962, 0.1962, 0.1287, 0.1287, 0, 0],[0, 0, -0.0071, 0.0071, 0.0001, 0.0039],[0.7137, 0.4156, 0.0606, 0.0606, 0, -1.8098],[0.4156, 0.7137, 0.0606, 0.0606, 0, 1.8098]],dtype = np.float)

self._CD = np.array([[1, -70, 0, 0],[1, 70, 0, 0],[0, 1, 0, 0],[0, 0, -0.25, 0.25]], dtype = np.float)

self._DD = np.array([[0, 0, 0, 0, 0, 0],[0, 0, 0, 0, 0, 0],[0, 0, 0, 0, 0, 0],[0, 0, 0, 0, 0, 0]], dtype = np.float)

def run(self):

while(True):

if self._state == S0_WAIT:

if self._shareEst.get() == True:

self._state = S1_RUN

elif self._state == S1_RUN:

u_star = np.array([[self.voltageL.get()],[self.voltageR.get()],[self.sL.get()],[self.sR.get()],[self.yaw.get()],[self.yaw_rate.get()]], dtype = np.float)

xhat_new = np.dot(self._AD, self._xhat)+np.dot(self._BD, u_star)

yhat = np.dot(self._CD, self._xhat) + np.dot(self._DD, u_star)

self._xhat = xhat_new

self._estS.put(float(xhat_new[0,0]))

self._estPsi.put(float(xhat_new[1,0]))

self._estOmegaL.put(float(xhat_new[2,0]))

self._estOmegaR.put(float(xhat_new[3,0]))

if self._shareEst.get() == False:

self._state = S0_WAIT

if not self.dataValsArc.full():

self.dataValsArc.put(float(xhat_new[0,0]))

if not self.dataValsPsi.full():

self.dataValsPsi.put(float(xhat_new[1,0]))

yield self._state