MEMORANDUM

| To: | Charlie Refvem, Lecturer, Department of Mechanical Engineering, Cal Poly SLO |  |
| --- | --- | --- |
|  | crefvem@calpoly.edu |  |
| From: | Quinn Patterson |
|  | qpatters@calpoly.edu |
|  | Jake Rowen |
|  | jhrowen@calpoly.edu |
| Team: Date: | 23 2/1/26 |  |
| RE: | Line Sensor Acquisition |  |

https://www.pololu.com/category/123/pololu-qtr-reflectance-sensors

We plan to order the line sensors between 1-2 weeks in advance. They will be purchased on Pololu’s website. There are overnight shipping options to SLO so there is plenty of leeway and possibility to order very late and still make it in time for week 7 lab.

Questions:

- What sensor interface is best? Analog? Digital? Serial? What are the tradeoffs?

Analog sensors will give us the best data, however analog pin on the board are needed, and if we decide to use more sensors than analog pins are available, we will need an ADC. Digital output is very simple and can wire into basically any GPIO pin, however, more would be needed to provide the same data quality as analog sensors. Serial communication arrays would work well if pins are a serial limitation, however, would potentially add latency which could make the data less useful for control. For this assignment analog sensors would be preferable.

- How many channels are best? How will the data be interpreted and used based on the number of channels?

The number of channels used depends on which sensor type we use, however for analog sensors 3-5 sensors would provide us with adequate data to track line position. If using an I2C array or digital sensors, more channels would provide better data, however line width would be a factor.

- What spacing between channels is best? Is tight packing better, or broad spacing?

A little bit of overlap using analog sensors is best. Packing too tightly would lead to a lot of data being redundant and spacing them too broadly may cause the sensors to miss a line if it’s too narrow.

- How far should the sensors be from the surface/line? What will happen if the sensor is closer or further from the line?

Distance from the surface depends on the sensor, however the Polulu QTR model sensors optimal height ranges from 5-20mm above the surface. If the sensors are too far from the line it will reduce the contrast and increase overlap between readings, and if they are too close it may increase sensitivity to bumps and create blind spots.

- Is "crossover" between channels a good or bad thing?

Crossover can be both a good and bad thing. Too much crossover makes sensors redundant and decreases the efficiency of our system. No crossover can cause blind spots and less consistent signaling.

- Are any of the above details dependent on how wide the line is?

Yes. If the line is narrower then we will need more resolution to detect the line and thus more sensors and pins.