MEMORANDUM

| To: | Charlie Refvem, Lecturer, Department of Mechanical Engineering, Cal Poly SLO |  |
| --- | --- | --- |
|  | crefvem@calpoly.edu |  |
| From: | Quinn Patterson |
|  | qpatters@calpoly.edu |
|  | Jake Rowen |
|  | jhrowen@calpoly.edu |
| Team: Date: | MECHA23 1/13/26 |  |
| RE: | Lab 0x00 Findings |  |

Table of Contents

- Abstract

- Figures

- Code Attachment

# Abstract

In this lab we familiarized ourselves with the NUCLEO-L476RG board and its basic functionalities. We learned how to flash firmware and upload micropython code through the Shoe of Brian. A simple RC circuit was then attached and the board was utilized to collect data during the circuits step response. From our data we determined our theoretical time constant to be 0.094s, and the measured constant to be 0.085s. This error comes out to about 12% and is close enough to be acceptable for our purposes.

# Figures

Figure 1. Theoretical and Experimental Step Responses

Figure 1 showcases our step response compared to the theoretical max. Since the capacitor is not ideal it can never truly reach its complete maximum, which is visualized in the plot.

Figure 2. Linearized Step Response with Line of Best Fit

Figure 2 showcases our linearized experimental data and a line of best fit we use to calculate the time constant. The line of best fit was calculated off the first 20% of the data because the spread after that begins to widen and can throw off our data.

# Code Attachments

- Measure and record step response

import pyb

from pyb import Pin

from pyb import ADC

from pyb import Timer

from array import array

from time import sleep

PC0 = Pin(Pin.cpu.C0, Pin.IN)

PC1 = Pin(Pin.cpu.C1, Pin.OUT)

adc = ADC(PC0)

tim = Timer(6, freq = 500)

data = array('H', [0] * 300)

idx = 0

done = False

PC1.low()

def tim_cb(tim):

global data, idx, done

val = adc.read()

if idx == 1:

PC1.high()

if idx < len(data):

data[idx] = val

idx += 1

else:

tim.callback(None)

done = True

# Toggle input and measure step response with ADC

tim.callback(tim_cb)

while not done:

pass

for idx, val in enumerate(data):

print(f"{idx/500}, {val*(3.3/4095)}")

PC1.low()

- Plot data from CSV file

from matplotlib import pyplot as plt

import math

import numpy as np

from numpy import array

with open('lab1_data.csv', 'r') as data:

items = data.readlines()

headers = items[0].strip().split(',')

data = []

for i in range(1, len(items)):

values = items[i].strip().split(',')

subvalues = []

for val in values:

if '#' in val:

subvals = val.partition('#')

for subval in subvals:

if subval.strip() != '':

subvalues.append(subval.strip())

else:

subvalues.append(val)

data.append(subvalues)

clean_data = []

for i in range(len(data)):

row = []

for j in range(len(data[i])):

try:

row.append(float(data[i][j]))

except ValueError:

if len(data[i])<= 2 and (len(data[i]) == 0 or data[i][j] == ''):

print(f"Empty line found on line {i+2}, skipping line.")

continue

elif "#" in data[i][j]:

print(f"Comment found on line {i+2}")

continue

else:

print(f"Invalid data on line {i+2}, skipping line.")

continue

if len(row) >= len(headers):

clean_data.append(row)

time = [row[0] for row in clean_data]

height = [row[1] for row in clean_data]

# time constant

step_vlt = .632*height[-1]

tau = 0.08461548312758245

theoretical = [0]*len(time)

for i in range(len(time)):

theoretical[i] = 3.3*(1 - math.exp(-1*time[i]/tau))

plt.plot(time, height, label= 'Experimental',marker='o')

plt.title('RC Curcuit Step Response')

plt.xlabel(headers[0])

plt.ylabel(headers[1])

plt.grid()

plt.plot(time, theoretical, label='Theoretical', color='red')

plt.legend()

plt.show()

# linearized step response

linearized = [0]*len(height)

for i in range(len(height)):

linearized[i] = math.log(abs(1-(height[i]/(max(height)+.000001))))

# best fit data

coefficients = np.polyfit(time[0:60], linearized[0:60], 1)

best_fit = [coefficients[0]*t + coefficients[1] for t in time]

print("Time constant from best fit: ", -1/coefficients[0])

plt.plot(time, linearized, label='Linearized Experimental', linestyle='None', marker='o')

plt.title('Linearized RC Circuit Step Response')

plt.xlabel('Time (s)')

plt.ylabel('Linearized Voltage')

plt.plot(time, best_fit, label='Best Fit Line', color='red')

plt.grid()

plt.legend()

plt.show()